ROV Sensor System Review & Upgrade Recommendations

## 1. Summary of Sensors Used in Previous ROV (Orca)

| Sensor Type | Model Used | Waterproof | Use Case |
| --- | --- | --- | --- |
| Depth Sensor | MS5540C | ❌ No (used inside air chamber) | Depth estimation via internal pressure reading |
| IMU Sensor | MPU6050 (6-axis) | ❌ No | Pitch and roll orientation, yaw drifted |
| Temperature | LM35 | ❌ No | Internal electronics temperature monitoring |
| Leak Sensor | Simple Detector | ✅ Yes | Leak detection inside main electronics housing |

## 2. Limitations in the Old Version

### Depth Sensing Issues:

- MS5540C was not waterproof and depended on sealed chamber pressure.

- Vulnerable to errors under long-term use, potential chamber sealing issues.

- Not suitable for deeper or extended dives.

### Orientation Issues (IMU):

- MPU6050 lacks a magnetometer, resulting in yaw drift, especially on longer dives.

- Inconsistent performance under vibration from motors/thrusters.

### Temperature Monitoring:

- LM35 only used for internal monitoring; external water temperature was not monitored.

- Analog output prone to noise, no water protection.

### Leak Detection:

- Basic leakage detection was present, but no fail-safe response system was integrated (no auto shutdown or alert escalation).

## 3. Recommended Upgrades (By Importance)

### 1️⃣ Depth Sensor Upgrade (Critical for Safety & Control)

- ✅ Recommended: MS5837-30BA for full waterproofing and deep-water operation (up to 300 meters), or MS5837-02BA for shallow waters.

- 🎯 Why: Accurate, waterproof, reliable depth control. Enables fail-safe auto ascent, safer deep dives, and better mission stability.

- 📝 Extra Tip: For mid-range depth missions (≤100m), MS5540C remains usable but proper waterproofing (custom housing) is mandatory.

### 2️⃣ IMU Sensor Upgrade (Important for Navigation & Stability)

- ✅ Recommended: MPU9250 (9-axis) or MPU6500 (IMU-only upgrade).

- 🎯 Why: Adds a magnetometer for stable yaw control, reducing heading drift. Essential for long or piloted missions where directional control matters.

- 📝 Extra Tip: Install magnetometer away from thruster magnetic fields. Use proper filtering (Madgwick, Mahony).

### 3️⃣ Temperature Sensing (Important for Environmental Data)

- ✅ Recommended:

  - Internal: Keep LM35 or upgrade to DS18B20 (internal) if noise is a concern.

  - External (Water): Add DS18B20 Waterproof or Blue Robotics Celsius Sensor for accurate external temperature profiling.

- 🎯 Why: Supports environmental studies, thermal profiling, and system safety.

### 4️⃣ Leakage Detection (Important for System Protection)

- ✅ Recommended: Keep existing Leak Sensor, but improve:

  - Add multiple leak detection points

  - Link leak detection to automatic shutdown or alert system

- 🎯 Why: Redundancy in leak detection prevents mission loss and hardware damage.

## 4. Final Summary Table

| System Function | Old Sensor | Issue | Recommended Sensor |
| --- | --- | --- | --- |
| Depth Control | MS5540C | ❌ Not waterproof, indirect reading | ✅ MS5837-30BA / 02BA |
| Orientation Control | MPU6050 | ❌ Yaw drift | ✅ MPU9250 (yaw correction) |
| Temperature (internal) | LM35 | ✅ OK for internal, analog noise | ✅ DS18B20 (digital), optional |
| Temperature (external) | None | ❌ Not monitored | ✅ DS18B20 / Blue Robotics |
| Leak Detection | Simple Detector | ✅ Present but basic | ✅ Expand detection points |
| Extra Environmental | None | ❌ Not available | ✅ Optional: pH, turbidity, etc. |

## 5. Conclusion

✔️ Priority upgrades: Depth Sensor → IMU → External Temperature → Leak Detection Improvements.

✔️ Why: These directly impact safety, maneuverability, and data quality for underwater missions.

✔️ Optional Upgrades: Environmental sensors can be added based on mission goals and budget.